Edge AI Accelerator IP

Design Documentation

Version 18.4  ·  Weight-Stationary Systolic Array with Structured Sparsity

accel_top_v18  ·  accel_pkg_v18  ·  AXI4-Lite + AXI4-Stream

# 1. Top-Level Architecture

## 1.1 Block Diagram

The top-level module accel_top_v18 integrates all sub-systems. The diagram below shows the primary data and control paths:

## 1.2 Data Flow

### 1.2.1 Weight Streaming Path

Weights arrive on the 128-bit AXI4-Stream weight slave interface (s_axis_weight_*). Each 128-bit beat carries six 20-bit weight packets (two 8-bit weights + two 2-bit sparse indices per packet, structured as weight_stream_128_t). The TUSER sideband carries 8 bits of sparsity metadata in the weight_tuser_t struct: a 4-bit sparse_mask, a 2-bit sparsity_mode, a last_in_tile flag, and a valid flag.

axis_weight_rx_v18 receives the stream, reassembles multi-beat rows (2 beats for 8 columns: 6+2 packets), and presents one complete row of COLS_P weight packets to the weight_tile_buffer_v18. The buffer holds up to WFIFO_DEPTH (default 8) complete tiles of ROWS_P rows each. On FSM command, each row is presented on wgt_in[COLS_P] to the systolic array during the S_LOAD_WEIGHTS phase.

### 1.2.2 Activation Streaming Path

Activations arrive on the 128-bit AXI4-Stream activation slave interface (s_axis_act_*). Each beat carries 16 INT8 activations (activation_stream_128_t). axis_activation_rx_v18 uses a 2-entry skid buffer to handle backpressure and presents the first 32-bit activation packet (4 activations, ACT_PKT_WIDTH=32) to the skew_buffer_v18.

The skew_buffer_v18 staggers the single activation packet across ROWS_P delay lines: Row 0 receives act_in directly; Row r receives act_in delayed by r × PE_STAGES (2) cycles. This skew aligns each activation packet with the correct row of the weight-stationary array so that the activation wavefront propagates diagonally, matching the weight data already resident in the PEs.

### 1.2.3 Compute Phase (Systolic Array)

The systolic_array_v18 implements an 8×8 mesh of pe_v18 processing elements. Each PE stores its weight packet (two weights + two sparse indices) during the S_LOAD_WEIGHTS phase when load_en is asserted row-by-row under load_row_sel. During the S_STREAM phase, compute_en is asserted and activations propagate horizontally across each row. Partial sums (psums) flow vertically downward, accumulating through each column.

Each PE executes a dual MAC per cycle: psum_out = psum_in + (w0 × a0) + (w1 × a1). The pipeline has PE_STAGES=2 register stages (multiply in Stage 1, accumulate in Stage 2). Saturating addition prevents accumulator overflow.

### 1.2.4 Output Path

Column outputs from the bottom row of the array are skewed diagonally in time (Column 0 finishes first, Column COLS_P-1 last) due to the activation wavefront propagation. The deskew_buffer_v18 re-aligns all columns by delaying earlier-completing columns by (COLS_P-1-c) × PE_STAGES cycles, producing a synchronised COLS_P-wide output vector.

The aligned result passes through the activation_func_v18 (ReLU, ReLU6, Leaky ReLU, or bypass) and then through the 3-stage vector_postproc_v18 pipeline (Bias → Scale → Shift+Round+Saturate). The final result is stored in the output_collector_fifo_v18 (512 entries, FWFT). The axis_result_tx_v18 drains the FIFO, packing 4 × 32-bit results per 128-bit TDATA beat, and asserts TLAST at the end of each drain_count output vectors.

## 1.3 Structured Sparsity Hardware

Structured sparsity is encoded in the weight stream itself. Each weight packet (WGT_PKT_WIDTH = 20 bits) carries two non-zero weights and two 2-bit indices indicating which of the four input activations each weight should multiply. The TUSER field sparsity_mode (2 bits) propagates the mode to the control logic via CSR_SPARSITY.

Inside each pe_v18, the activation selection mux (sel_a0, sel_a1) uses idx0_reg and idx1_reg to pick the two correct activations out of the four-element act_packet_in:

- SPARSITY_DENSE (2'b00): Both phases are used. In Phase 0 (stall_phase=0), activations a0 and a1 are selected; in Phase 1 (stall_phase=1), activations a2 and a3 are selected. Each vector requires two clock phases.

- SPARSITY_2_4 (2'b01): 50% sparsity. Two non-zero weights per group of four. idx0_reg selects the activation for w0; idx1_reg selects the activation for w1. Single-phase accumulation.

- SPARSITY_1_4 (2'b10): 75% sparsity. Only one non-zero weight per group of four. idx0_reg selects the activation for w0; the second weight (w1) and its index are unused. sel_a1 remains zero.

- SPARSITY_4_8 (2'b11): 50% sparsity over an 8-element block. Operates identically to 2:4 in hardware terms but at a different granularity. Both indices are used.

Zero-weight detection (zero_weight_detected) and zero-activation detection (zero_act_detected) gate the operand isolate_operands signal. When either is true, operands iso_w0, iso_w1, iso_a0, and iso_a1 are forced to zero, preventing any switching activity in the multiplier and saving dynamic power. This is distinct from sparsity skipping: the index mux always executes the correct activation selection, but the multiplication itself is suppressed when both weights are zero.

The per-PE clock gating cell (icg_cell) further cuts dynamic power by gating gated_clk when neither load_en nor compute_en is asserted (datapath_active = load_en | compute_en).

# 2. Module Breakdown

## 2.1 Package: accel_pkg_v18  (01_pkg_v18.sv)

The package defines all shared type definitions, localparams, enumerated types, and CSR address constants used across the design. It is the single source of truth for all configurable dimensions.

| Symbol | Value | Description |
| --- | --- | --- |
| ROWS / COLS | 8 | Systolic array dimensions |
| PE_STAGES | 2 | Pipeline depth per PE |
| W_WIDTH / A_WIDTH | 8 | Weight and activation bit widths |
| ACC_WIDTH | 32 | Accumulator width (INT32) |
| IDX_WIDTH | 2 | Sparse index width |
| WGT_PKT_WIDTH | 20 | 2×W + 2×IDX per weight packet |
| ACT_PKT_WIDTH | 32 | 4×A per activation packet |
| AXIS_DATA_WIDTH | 128 | AXI-Stream TDATA width (V18.4) |
| AXIS_USER_WIDTH | 8 | TUSER width (sparsity metadata) |
| AXIL_ADDR_WIDTH | 12 | AXI-Lite address width (4 KB CSR) |
| AXIL_DATA_WIDTH | 32 | AXI-Lite data width |
| OUTPUT_FIFO_DEPTH | 512 | Output collector FIFO depth |
| WFIFO_DEPTH | 8 | Weight tile buffer depth |
| PP_BIAS_WIDTH | 16 | Per-channel bias width |
| PP_SCALE_WIDTH | 16 | Scale multiplier width |
| PP_SHIFT_WIDTH | 6 | Right-shift amount (0–63) |
| PP_PIPELINE_STAGES | 3 | Post-processing pipeline depth |

Key enumerated types defined in the package:

- sparsity_mode_e: SPARSITY_DENSE=2'b00, SPARSITY_2_4=2'b01, SPARSITY_1_4=2'b10, SPARSITY_4_8=2'b11

- compute_state_e: S_IDLE, S_LOAD_WEIGHTS, S_STREAM, S_DRAIN, S_DONE, S_ERROR, S_RECOVERY

- activation_e: ACT_NONE, ACT_RELU, ACT_RELU6, ACT_LEAKY_RELU, ACT_SIGMOID, ACT_TANH

- postproc_op_e: PP_NONE, PP_BIAS_ADD, PP_SCALE_SHIFT, PP_REQUANT, PP_BIAS_SCALE

- pe_power_state_e: PE_ACTIVE, PE_IDLE, PE_CLOCK_GATE, PE_POWER_DOWN

- weight_tuser_t: Packed struct: {sparse_mask[3:0], sparsity_mode[1:0], last_in_tile, valid} = 8 bits

## 2.2 Module: pe_v18  (02_core_and_array_v18.sv)

The Processing Element is the fundamental compute unit. It stores one weight packet stationary (weight-stationary dataflow) and performs a dual MAC against streaming activations chosen by its index registers.

Key Ports

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| wgt_packet_in | in | WGT_PKT_W=20 | Weight packet {idx1, idx0, w1, w0} |
| wgt_parity_in / parity_err_in/out | in/out | 1 | Parity chain for error propagation |
| act_packet_in / act_packet_out | in/out | ACT_PKT_W=32 | 4-activation packet, horizontal pass-through |
| psum_in / psum_out | in/out | ACC_W=32 (signed) | Partial sum vertical dataflow |
| psum_valid_in / psum_valid_out | in/out | 1 | Valid flag travelling with psum |
| load_en | in | 1 | Load weight packet from wgt_packet_in |
| compute_en | in | 1 | Enable MAC pipeline |
| stall_phase | in | 1 | Dense mode phase select (0=a0/a1, 1=a2/a3) |
| mode_dense | in | 1 | 1=dense two-phase mode, 0=sparse |
| sparsity_cfg | in | sparsity_mode_e | Sparsity pattern select |
| zero_weight_detected | out | 1 | Both stored weights are zero |
| zero_act_detected | out | 1 | Both selected activations are zero |
| overflow_detected | out | 1 | Accumulator overflow in Stage 2 |
| power_state | out | pe_power_state_e | Current power state |
| mac_active | out | 1 | MAC actually performed this cycle |

Internal Pipeline

Stage 1 (posedge gated_clk): product_s1 = (iso_w0 × iso_a0) + (iso_w1 × iso_a1). Activation source is muxed by idx0_reg/idx1_reg in sparse mode, or stall_phase in dense mode.

Stage 2 (posedge gated_clk): In sparse mode, acc_reg = psum_s1 + product_s1 with saturation. In dense mode, Phase 0 stores to dense_partial; Phase 1 accumulates dense_partial + psum_s1 + product_s1. In dense Phase 0, psum_out is driven to zero to prevent stale data corrupting the downstream PE's psum chain.

Clock Gating

An icg_cell (latch-based glitch-free ICG) gates gated_clk when datapath_active = load_en | compute_en is de-asserted. The DFT scan_enable bypasses the latch during scan mode.

## 2.3 Module: systolic_array_v18  (02_core_and_array_v18.sv)

Instantiates ROWS_P × COLS_P PE instances with correct inter-PE wiring, and manages the horizontal/vertical pipeline alignment signals.

Key Ports

| Signal | Dir | Description |
| --- | --- | --- |
| load_en | in | Assert to load weights; qualified by load_row_sel |
| load_row_sel | in | log2(ROWS) bits: selects which row's PEs load wgt_in |
| compute_en | in | Enables MAC in all PEs |
| stall_phase | in | Dense mode phase toggle |
| input_valid | in | Valid activation data at row 0 |
| wgt_in[COLS_P] | in | One weight packet per column |
| act_in[ROWS_P] | in | Skewed activation packets, one per row |
| psum_out[COLS_P] | out | Raw column sums from bottom row |
| col_valid[COLS_P] | out | Per-column valid flags (gated by phase in dense mode) |
| result_valid | out | col_valid[0] (global result valid) |
| parity_error | out | OR of all column parity chain errors |
| overflow_error | out | OR of all per-column overflow bits |
| pe_mac_active_map[ROWS×COLS] | out | Flat bitmap of active PEs for perf counters |
| pe_zero_weight_map / pe_zero_act_map | out | Zero-operand detection maps for perf counters |
| active_pe_count | out | Popcount of pe_mac_active_map |

Skew/deskew alignment: valid_v[0][c] is driven by input_valid (col 0) or valid_h_pipe[2c-1] (other columns) to align the valid handshake with the skewed activation wavefront. stall_phase is similarly pipelined both horizontally and vertically (stall_phase_h_pipe and stall_phase_v_pipe) so each PE sees its phase aligned with its incoming valid psum.

## 2.4 Modules: skew_buffer_v18 / deskew_buffer_v18  (02_core_and_array_v18.sv)

skew_buffer_v18: Inserts row r an r × PE_STAGES = 2r cycle delay on the activation packet, creating the diagonal wavefront required for weight-stationary systolic operation. Row 0 is a direct wire; rows 1–7 use shift registers of depth 2, 4, 6, 8, 10, 12, and 14 cycles respectively.

deskew_buffer_v18: Re-aligns column outputs. Column c is delayed by (COLS_P − 1 − c) × PE_STAGES cycles. Column 7 (last) has zero delay; column 0 (first) has the maximum delay of 14 cycles. Both valid and psum signals are shifted together. The deskew runs whenever enable (compute_en | drain_en) is asserted or there is valid data in the pipeline.

## 2.5 Module: activation_func_v18  (02_core_and_array_v18.sv)

One pipeline register stage applied to each of COLS_P output values. The func_sel (activation_e) input chooses from ACT_NONE (bypass), ACT_RELU (max(0,x)), ACT_RELU6 (min(max(0,x), 6<<8)), or ACT_LEAKY_RELU (alpha ≈ 0.125 via arithmetic right-shift by 3). ACT_SIGMOID and ACT_TANH are defined in the enum but are LUT-based and marked [TODO] in this implementation. The valid flag is unconditionally registered regardless of the data mux to maintain correct timing.

## 2.6 Module: weight_tile_buffer_v18  (03_buffers_v18.sv)

A dual-port tile buffer that decouples the AXI-Stream weight receiver from the systolic array loading sequence. It stores up to TILE_DEPTH (default 8) complete weight tiles, each containing ROWS_P rows, each row being COLS_P × WGT_PKT_WIDTH bits wide.

Write Interface (from axis_weight_rx_v18)

- wr_start: Pulse to begin a new tile. The W_IDLE → W_LOADING transition samples row 0 on the same cycle if wr_valid is also asserted.

- wr_valid + wr_data[COLS_P]: Each asserted cycle writes one row to the current tile. After ROWS_P rows, the W_COMMIT state increments wr_tile_ptr and asserts wr_done.

- wr_ready: De-asserted when tile_cnt == TILE_DEPTH (buffer full).

Read Interface (to compute_controller_v18)

- rd_start: Transitions R_IDLE → R_STREAMING. Only possible when has_tile_ready.

- rd_next_row: Advances rd_row_cnt. After ROWS_P rows, transitions to R_DONE awaiting rd_done.

- rd_valid / rd_last_row: Active during R_STREAMING; rd_last_row indicates the final row of the tile.

- can_accept_tile: Combinational: tile_cnt < TILE_DEPTH. Used by the controller for prefetch decisions (wgt_prefetch_start, V18.3).

## 2.7 Module: axis_weight_rx_v18  (04_axis_interfaces_v18.sv)

Receives the 128-bit weight AXI-Stream, reassembles multi-beat rows (PKTS_PER_BEAT = 128/20 = 6 packets per beat; BEATS_PER_ROW = ceil(COLS_P/6) = 2 beats for COLS_P=8), and outputs a complete row to the weight tile buffer.

The TUSER byte is decoded as weight_tuser_t to extract sparsity_mode and sparse_mask. The wgt_parity[COLS_P] output is generated by XOR-folding each assembled weight packet. s_axis_tready is de-asserted when the tile buffer is not ready (wgt_tile_ready=0) or when not in a receiving state.

## 2.8 Module: axis_activation_rx_v18  (04_axis_interfaces_v18.sv)

Receives the 128-bit activation AXI-Stream and presents the first 32-bit slice (act_data = s_axis_tdata[31:0]) to the skew buffer. A 2-entry skid buffer absorbs backpressure: if act_ready is de-asserted while a beat is accepted, the 32-bit packet and tlast are stored in the skid register and replayed when the downstream is ready. rx_done is pulsed on TLAST handshake.

## 2.9 Module: axis_result_tx_v18  (04_axis_interfaces_v18.sv)

Reads from the output_collector_fifo_v18 and transmits results on the 128-bit AXI-Stream result master interface. It packs RESULTS_PER_BEAT=4 results (4 × 32-bit = 128-bit) per TDATA beat. For COLS_P=8, BEATS_PER_ROW=2 beats are sent per output vector. drain_start is driven combinationally by compute_en; drain_count is the configured vector_count. TLAST is asserted on the final beat of the final vector. TKEEP is always 16'hFFFF (all bytes valid). TUSER is unused on the result path (driven to 0).

## 2.10 Module: accel_csr_axil_v18  (05_control_v18.sv)

Implements the AXI4-Lite slave interface for all CSR registers. Independent write and read state machines (AXIL_IDLE → AXIL_WRITE → AXIL_RESP and AXIL_IDLE → AXIL_READ → AXIL_RESP) handle the separate AW+W and AR channels. Write responses are OKAY for all implemented addresses; unimplemented addresses return 32'hDEAD_BEEF on reads.

Self-clearing pulse outputs (ctrl_start, ctrl_soft_reset, ctrl_abort, ctrl_clear) are asserted for exactly one clock cycle when the corresponding bit is written to CSR_CTRL and are cleared at the start of each always_ff block. The scalable bias memory (bias_mem[COLS_P]) can be populated either through legacy CSR_PP_BIAS_0–7 writes (two 16-bit biases per register) or through the V18.3 CSR_PP_BIAS_ADDR / CSR_PP_BIAS_DATA auto-increment interface.

## 2.11 Module: compute_controller_v18  (05_control_v18.sv)

The main compute FSM drives all array control signals (load_en, compute_en, drain_en, stall_phase, input_valid) and co-ordinates the weight tile buffer read-side. It also implements weight reuse (V18.2) and weight prefetch (V18.3).

| State | Entry Condition | Activities |
| --- | --- | --- |
| S_IDLE | Reset or all tiles done | Waits for enable && start. If wgt_tile_ready, goes to S_LOAD_WEIGHTS; else S_STREAM |
| S_LOAD_WEIGHTS | start && wgt_tile_ready | Asserts load_en row-by-row using wgt_row_valid and load_row_sel. Exits when wgt_last_row |
| S_STREAM | Weight loaded | Asserts compute_en, input_valid (when act_valid && !backpressure). stall_phase toggles each cycle for dense mode. Exits when inputs_done |
| S_DRAIN | inputs_done | Maintains compute_en for PIPELINE_LATENCY cycles to flush the array pipeline. drain_cnt counts up |
| S_DONE | pipeline_drained | If act_tile_cnt < act_tile_count: stay on weights, go to S_STREAM. Else if more tiles: S_LOAD_WEIGHTS. Else S_IDLE |
| S_ERROR | abort asserted | Sticky error state. Clears on ctrl_soft_reset |
| S_RECOVERY | [TODO] internal | Counts 32 cycles then returns to S_IDLE |

PIPELINE_LATENCY is computed as: 2 (input pipeline) + (ROWS_P − 1) (skew) + (ROWS_P × PE_STAGES) (vertical PE propagation) + ((COLS_P − 1) × PE_STAGES) (deskew) + 1 (activation function) + 8 (margin). For the default 8×8 array this evaluates to 2 + 7 + 16 + 14 + 1 + 8 = 48 cycles.

Weight prefetch (V18.3): While in S_STREAM or S_DRAIN, if !prefetch_issued && need_more_tiles && wgt_can_accept, the controller asserts wgt_prefetch_start for one cycle, instructing the AXI weight receiver to begin streaming the next tile into the buffer while the current tile is still being processed.

## 2.12 Module: vector_postproc_v18  (07_postproc_v18.sv)

A 3-stage fully-pipelined post-processing unit operating on COLS_P parallel data paths:

- Stage 1 – Bias Addition: data_in[c] + sign_extend(bias[c]) with optional saturation for PP_BIAS_ADD, PP_REQUANT, and PP_BIAS_SCALE modes.

- Stage 2 – Scale Multiplication: s1_data[c] × scale (full-precision, EXT_W = DATA_W + SCALE_W + 1 = 49 bits) for PP_SCALE_SHIFT, PP_REQUANT, and PP_BIAS_SCALE modes. Non-scaling modes sign-extend for bypass.

- Stage 3 – Shift + Round + Saturate: Optional round_en adds 0.5 before arithmetic right shift by pp_shift (0–63). sat_en clips to [sat_min, sat_max]. sat_flag[c] indicates clipping. Output is truncated to DATA_W=32 bits.

All pipeline controls (op_sel, round_en, sat_en, shift, scale, sat_max, sat_min) are registered at each stage boundary so the configuration is coherent through the pipeline.

## 2.13 Module: perf_counters_v18  (05_control_v18.sv)

Five 32-bit saturating counters track hardware activity while ctrl_enable is high:

- cnt_cycles: increments every cycle that compute_en is asserted

- cnt_stalls: increments when result_fifo_full && compute_en (output backpressure)

- cnt_macs: popcount(pe_mac_active_map) per cycle — counts actual MACs executed

- cnt_zero_weights: popcount(pe_zero_weight_map) per cycle — zero-weight skips

- cnt_zero_acts: popcount(pe_zero_act_map) per cycle — zero-activation skips

All counters are cleared when ctrl_clear (CTRL_FLUSH_BIT) is written to CSR_CTRL. They are readable via AXI-Lite as CSR_PERF_CYCLES through CSR_PERF_ZA.

## 2.14 Module: irq_controller_v18  (05_control_v18.sv)

Rising-edge detector on eight IRQ sources (irq_sources), latches pulses into irq_pending, and asserts irq_out = |(irq_pending & irq_mask) && global_enable. IRQ bits are cleared by writing 1s to CSR_IRQ_STATUS (write-1-to-clear). IRQ sources in priority order:

| Bit | Name | Source |
| --- | --- | --- |
| 0 (IRQ_COMPUTE_DONE) | done | Computation fully complete (sticky done flag) |
| 1–2 | reserved | — |
| 3 (IRQ_UNDERFLOW) | wfifo_empty | Weight FIFO became empty unexpectedly |
| 4 | afifo_empty | Activation FIFO empty (act_valid=0) |
| 5 (IRQ_OVERFLOW) | rfifo_full | Output FIFO full (output backpressure) |
| 6 (IRQ_ECC_UNCORR) | error_flag | FSM reached S_ERROR |
| 7 | reserved | — |

# 3. Protocol Descriptions

## 3.1 AXI4-Lite Interface

The accelerator exposes a 12-bit address, 32-bit data AXI4-Lite slave port (s_axil_*) for all register access. The 4 KB address space (AXIL_ADDR_WIDTH=12) accommodates future expansion.

### 3.1.1 Write Handshake

The CSR block implements independent write-address and write-data channels. Both AWREADY and WREADY are asserted simultaneously in AXIL_IDLE. When both AWVALID and WVALID are seen, address and data are latched in one cycle and the state machine moves to AXIL_WRITE. The write decode completes in AXIL_WRITE (do_write pulse); BRESP OKAY is returned and the state moves to AXIL_RESP where it waits for BREADY. There is no error response for writes to unimplemented addresses (writes are silently ignored).

### 3.1.2 Read Handshake

ARREADY is asserted in AXIL_IDLE. On ARVALID, the address is latched and the state moves to AXIL_READ (1 cycle), then asserts RVALID with the combinational rdata_mux in AXIL_RESP, waiting for RREADY. Unimplemented addresses return 32'hDEAD_BEEF. All accesses have 2-cycle read latency (AR → R).

### 3.1.3 CSR Register Map

| Address | Register | Access | Description |
| --- | --- | --- | --- |
| 0x000 | CSR_CTRL | R/W | Control: [0]=enable, [1]=start*, [2]=soft_rst*, [4]=abort*, [6]=flush* (*self-clearing) |
| 0x004 | CSR_STATUS | RO | [0]=busy, [1]=done, [2]=error, [10:8]=state[2:0] |
| 0x008 | CSR_TILE_CFG | R/W | [7:0]=vector_count (N vectors), [31:16]=tile_count (M weight tiles) |
| 0x00C | CSR_SPARSITY | R/W | [0]=mode_dense, [2:1]=sparsity_mode, [5:3]=activation_fn |
| 0x010 | CSR_IRQ_EN | R/W | [7:0] IRQ enable mask |
| 0x014 | CSR_IRQ_STATUS | R/W1C | [7:0] IRQ status (write 1 to clear) |
| 0x018 | CSR_IRQ_FORCE | WO | Force IRQ for test [TODO: not connected in V18] |
| 0x020 | CSR_AXIS_WFIFO | RO | Weight FIFO: [4:0]=level, [6]=full, [7]=empty |
| 0x024 | CSR_AXIS_AFIFO | RO | Activation FIFO: [4:0]=level, [6]=full, [7]=empty |
| 0x028 | CSR_AXIS_RFIFO | RO | Result FIFO: [4:0]=level, [6]=full, [7]=empty |
| 0x030 | CSR_PERF_CYCLES | RO | Total compute cycles |
| 0x034 | CSR_PERF_STALL | RO | Stall cycles (output backpressure) |
| 0x038 | CSR_PERF_MAC | RO | Actual MAC operations (PE-cycles) |
| 0x03C | CSR_PERF_ZW | RO | Zero-weight skips (PE-cycles) |
| 0x040 | CSR_PERF_ZA | RO | Zero-activation skips (PE-cycles) |
| 0x050 | CSR_WDOG_CFG | R/W | [0]=wdog_en, [1]=auto_rst, [31:8]=timeout [TODO] |
| 0x060 | CSR_ECC_CFG | R/W | [0]=ecc_en, [1]=corr_en, [2]=inj_en [TODO] |
| 0x0A0 | CSR_ACT_CFG | R/W | [2:0]=activation function override |
| 0x0B0 | CSR_PP_CTRL | R/W | [2:0]=pp_op_sel, [8]=round_en, [9]=sat_en |
| 0x0B4 | CSR_PP_SCALE | R/W | [15:0]=scale multiplier (signed) |
| 0x0B8 | CSR_PP_SHIFT | R/W | [5:0]=right-shift amount (0–63) |
| 0x0BC | CSR_PP_SAT_MAX | R/W | Saturation maximum (32-bit signed) |
| 0x0C0 | CSR_PP_SAT_MIN | R/W | Saturation minimum (32-bit signed) |
| 0x0C4–0x0E0 | CSR_PP_BIAS_0–7 | R/W | Packed bias registers: 2×16-bit per register (bias[0..15]) |
| 0x0E4 | CSR_ACT_TILE_CFG | R/W | [31:16]=act_tile_count (activation tiles per weight load, default=1) |
| 0x0E8 | CSR_PP_BIAS_ADDR | R/W | [7:0]=column index for next scalable bias write |
| 0x0EC | CSR_PP_BIAS_DATA | WO | [15:0]=bias value; auto-increments CSR_PP_BIAS_ADDR |
| 0x0F0 | CSR_CAP0 | RO | {PE_STAGES[31:24], rsvd[23:18], sparse_modes[17:16], COLS[15:8], ROWS[7:0]} |
| 0x0F4 | CSR_CAP1 | RO | Buffer depths: {WFIFO[31:24], RFIFO[23:16], AFIFO[15:8], WFIFO_DEP[7:0]} |
| 0x0F8 | CSR_CAP2 | RO | Feature bits: ECC[0], Debug[1], Watchdog[2], ClkGate[3], Sparsity[4], PP[5], 128b-AXI[6] |
| 0x0FC | CSR_VERSION | RO | IP version: {MAJOR[31:24], MINOR[23:16], PATCH[15:0]} |

## 3.2 AXI4-Stream Interfaces

### 3.2.1 Weight Stream (Slave, 128-bit)

Signals: s_axis_weight_tdata[127:0], s_axis_weight_tkeep[15:0], s_axis_weight_tlast, s_axis_weight_tuser[7:0], s_axis_weight_tvalid, s_axis_weight_tready.

TDATA packing (weight_stream_128_t): Six 20-bit weight packets occupy bits [119:0]. Each packet is {idx1[1:0], idx0[1:0], w1[7:0], w0[7:0]}. Bits [127:120] are reserved (unused in V18.4).

TKEEP: All 16 bytes should be valid (16'hFFFF) for full beats. For a partial last beat (e.g., columns 7:6 in the second beat), only the relevant bytes need to be marked valid. The hardware currently uses the weight packet index rather than TKEEP to detect validity.

TLAST semantics: TLAST is asserted on the last beat of the last row of the last column of a tile. The weight_tuser_t.last_in_tile flag in TUSER is the recommended mechanism; TLAST is also used as a fallback trigger for tile completion in axis_weight_rx_v18.

TUSER (weight_tuser_t, 8 bits): [7:4]=sparse_mask (which of 4 weights are non-zero), [3:2]=sparsity_mode (maps to sparsity_mode_e), [1]=last_in_tile, [0]=valid. The hardware extracts sparsity_mode to propagate to the systolic array via sparsity_cfg.

### 3.2.2 Activation Stream (Slave, 128-bit)

Signals: s_axis_act_tdata[127:0], s_axis_act_tkeep[15:0], s_axis_act_tlast, s_axis_act_tvalid, s_axis_act_tready. No TUSER on the activation stream.

TDATA packing (activation_stream_128_t): 16 × INT8 activations packed as act15[127:120] … act0[7:0]. Only the first 32-bit slice (act3[31:24]…act0[7:0]) is consumed per beat by the current axis_activation_rx_v18 implementation (simplified pass-through).

TLAST semantics: TLAST marks the end of one activation vector (or batch of vectors). The act_last signal is forwarded to the FSM. In the current design, TLAST triggers rx_done which is used for status, not for FSM state transitions (the FSM uses vector_cnt instead).

Flow control: s_axis_act_tready is asserted when enable is high and either no skid data is pending or the downstream (act_ready from the FSM) is ready. The 2-entry skid buffer ensures the producer is not stalled for more than 1 cycle during FSM state transitions.

### 3.2.3 Result Stream (Master, 128-bit)

Signals: m_axis_result_tdata[127:0], m_axis_result_tkeep[15:0], m_axis_result_tlast, m_axis_result_tuser[7:0], m_axis_result_tvalid, m_axis_result_tready.

TDATA packing (result_stream_128_t): 4 × INT32 results = {result3[127:96], result2[95:64], result1[63:32], result0[31:0]}. For COLS_P=8, two beats are required per output row.

TKEEP: Always 16'hFFFF. TUSER: Always 8'h00 (unused on result path). TLAST: Asserted on the final beat of the final output vector in the current drain sequence (row_cnt == drain_count − 1 && beat_cnt == BEATS_PER_ROW − 1).

Flow control: m_axis_result_tvalid is asserted when the TX FSM is in TX_SEND. If the downstream de-asserts m_axis_result_tready, the TX FSM stalls in TX_SEND (beat_cnt is not incremented). result_read (output FIFO pop) is only asserted on the first beat of each row (beat_cnt==0) and qualified with m_axis_tready, so rows are consumed from the FIFO in order without risk of underflow.

# 4. IP Capabilities

## 4.1 Sparsity Modes

| Mode | CSR_SPARSITY [0] (mode_dense) | CSR_SPARSITY [2:1] (sparsity_mode) | Weights used / group | Theoretical speedup |
| --- | --- | --- | --- | --- |
| Dense | 1 | — | 4 of 4 (two phases) | 1× |
| 2:4 | 0 | 2'b01 | 2 of 4 | Up to 2× vs dense |
| 1:4 | 0 | 2'b10 | 1 of 4 | Up to 4× vs dense |
| 4:8 | 0 | 2'b11 | 4 of 8 | Up to 2× vs dense |

Selection: Write CSR_SPARSITY. Bit [0]=1 forces dense mode regardless of bits [2:1]. Bit [2:1] selects among the three structured sparsity modes when bit [0]=0. The sparsity_mode is also carried in TUSER of the weight stream; axis_weight_rx_v18 forwards it to the systolic array, though CSR_SPARSITY takes precedence in the current RTL (the controller uses the CSR value directly via sparsity_cfg).

## 4.2 Performance Metrics (Default 8×8 Array)

| Metric | Dense | 2:4 Sparse | 1:4 Sparse |
| --- | --- | --- | --- |
| Peak MACs/cycle (array) | 64 dual-MACs = 128 MAC/cycle | 64 (one phase) | 32 (single weight per PE) |
| Theoretical Peak GOPS @ 1 GHz | 256 GOPS | 128 GOPS | 64 GOPS |
| Weight stream bandwidth | 20 b × 8 cols × 2 beats × freq | Same | Same |
| Activation bandwidth | 32 b / vector | 32 b / vector | 32 b / vector |
| Result bandwidth | 8 × 32 b = 256 b / vector | 256 b / vector | 256 b / vector |
| Utilisation under ideal backpressure | ~100% | ~50–100%* | ~25–100%* |

* Utilisation in sparse modes depends on the fraction of actual zero weights/activations detected. The pe_mac_active_map and pe_zero_weight_map counters expose per-PE activity for profiling.

## 4.3 Scalability

- Array dimensions: ROWS_P and COLS_P are top-level parameters (default 8×8). The design scales to any rectangular dimension within AXI-Stream bandwidth constraints. The skew/deskew delay chains and pipeline latency calculation automatically adjust.

- Data width: W_WIDTH, A_WIDTH, and ACC_WIDTH are localparams in accel_pkg_v18. Changing them requires re-deriving WGT_PKT_WIDTH and ACT_PKT_WIDTH and verifying the downstream arithmetic bitwidths.

- FIFO depths: OUTPUT_FIFO_DEPTH (512) and WFIFO_DEPTH (8) are package parameters and can be overridden at instantiation.

- Bias memory: CSR_PP_BIAS_ADDR / CSR_PP_BIAS_DATA provides an auto-incrementing write interface that supports any number of columns up to 256 (8-bit address). The fixed CSR_PP_BIAS_0–7 interface covers up to 16 columns.

## 4.4 Clock Gating and Low-Power Features

- Per-PE ICG: icg_cell (ENABLE_ICG=1 by default) gates each PE's pipeline registers when compute_en=0 && load_en=0. This is a glitch-free latch-based cell with scan_enable bypass.

- Operand isolation: isolate_operands (zero_weight_detected | zero_act_detected) forces multiplier inputs to zero, eliminating switching activity in the MAC without involving the clock gate.

- PE power states: pe_power_state_e is output per PE but not currently wired to external power control; it serves as a monitoring output for future retention/power-domain integration.

- CTRL_FLUSH_BIT: Writes a synchronous clear through all FIFOs and state machines, bringing the design to a quiescent state without full hard reset.

## 4.5 Error Handling

| Error Type | Detection Mechanism | Status Signal | Response |
| --- | --- | --- | --- |
| Weight parity error | XOR parity chain through PE column (ENABLE_PARITY=1) | parity_error (array output) | IRQ [TODO: connect to IRQ source] |
| Accumulator overflow | Signed overflow detection in PE Stage 2 saturation logic | overflow_error (array output) | Saturates to SAT_MAX/SAT_MIN; overflow_detected per PE |
| Post-processing saturation | sat_flag[c] in vector_postproc_v18 | postproc_sat_flag (internal) | Clips to sat_max/sat_min; flag available on wires [TODO: expose to CSR] |
| FSM abort | CTRL_ABORT_BIT written while busy | S_ERROR state, error_flag=1 | IRQ_COMPUTE_DONE stays low; IRQ[6] (error_flag) fires; clears on ctrl_soft_reset |
| Output FIFO full | ofifo_full asserted | CSR_AXIS_RFIFO[6], IRQ[5] | Backpressure: FSM stalls (backpressure=result_fifo_full gates compute_en) |
| FIFO overflow/underflow | sync_fifo_v18 overflow/underflow signals | overflow/underflow ports | [TODO: not routed to IRQ or CSR in V18] |
| Watchdog timeout | CSR_WDOG_CFG.wdog_en [TODO] | IRQ[7] | [TODO: watchdog counter not implemented in RTL] |
| ECC SRAM errors | CSR_ECC_CFG.ecc_en [TODO] | CSR_ECC_STATUS, IRQ[5/6] | [TODO: ECC encoder/decoder not present in V18 RTL] |

# 5. Configuration and Usage Flow

## 5.1 Step-by-Step Computation Sequence

The following sequence assumes a single weight tile and N activation vectors (dense or sparse mode):

- Reset: Assert rst_n low for at least 2 clock cycles, then de-assert. All FSM states return to S_IDLE, FIFOs clear, performance counters reset.

- Enable and configure: Write CSR_CTRL with bit [0] (CTRL_ENABLE_BIT) set. Write CSR_SPARSITY with the desired mode_dense, sparsity_mode, and activation_fn. For 2:4 sparse: write 0x00000002 (mode_dense=0, sparsity=SPARSITY_2_4).

- Program tile dimensions: Write CSR_TILE_CFG: [15:0]=vector_count (N), [31:16]=tile_count (number of weight tiles). For a single tile with 4 vectors, write 0x00010004.

- Configure post-processing: Write CSR_PP_CTRL with desired op_sel, round_en, sat_en. Write CSR_PP_SCALE, CSR_PP_SHIFT, CSR_PP_SAT_MAX, CSR_PP_SAT_MIN. Load per-channel biases by writing to CSR_PP_BIAS_ADDR=0 then writing N values to CSR_PP_BIAS_DATA (auto-increments address).

- Configure IRQs: Write CSR_IRQ_EN with desired enable mask (e.g., 0x01 for IRQ_COMPUTE_DONE only).

- Stream weights: Assert s_axis_weight_tvalid and present weight beats. The weight FIFO must be ready (CSR_AXIS_WFIFO[7]=0). Each beat contains 6 weight packets. For COLS_P=8, each row requires 2 beats. Set TUSER with the correct weight_tuser_t encoding. Assert TLAST on the final beat of the tile.

- Start computation: Write CSR_CTRL with bits [0] (enable) and [1] (start) both set. The FSM reads start as a 1-cycle pulse, checks wgt_tile_ready, and transitions to S_LOAD_WEIGHTS to begin loading row 0 from the weight tile buffer into the array.

- Stream activations: Once in S_STREAM, s_axis_act_tready is asserted by the activation receiver. Present activation vectors on s_axis_act_tdata. Each beat carries 16 INT8 activations, but the current receiver uses only the first 4 (act_data = tdata[31:0]). Assert TLAST at the end of each vector boundary.

- Poll or await IRQ: Poll CSR_STATUS bit [1] (STAT_DONE_BIT) for completion, or wait for an IRQ on the IRQ_COMPUTE_DONE source (bit 0). The FSM transitions S_STREAM → S_DRAIN → S_DONE → S_IDLE when vector_cnt reaches vector_count.

- Read results: Results are already streaming out on m_axis_result_* during the drain phase (drain_start = compute_en). The host DMA or consumer should accept beats as they arrive. TLAST marks the end of the result batch.

- Read performance counters: After done, read CSR_PERF_CYCLES, CSR_PERF_STALL, CSR_PERF_MAC, CSR_PERF_ZW, CSR_PERF_ZA. Optionally clear with CSR_CTRL bit [6] (CTRL_FLUSH_BIT) before the next run.

## 5.2 Multi-Vector Batching (Weight Reuse)

In the weight-stationary systolic array, the same weight tile can be reused across multiple activation tile batches without re-loading. This is configured via CSR_ACT_TILE_CFG[31:16] = act_tile_count (default 1).

When act_tile_count > 1:

- After each S_DONE, the FSM checks: if act_tile_cnt + 1 < act_tile_count, it increments act_tile_cnt, resets vector_cnt and drain_cnt, and re-enters S_STREAM. The PE array retains its loaded weights since load_en is not re-asserted.

- Only when act_tile_cnt reaches act_tile_count does the FSM advance tile_cnt and reload the next weight tile (S_LOAD_WEIGHTS) or declare all-done (S_IDLE).

- The host must stream the next set of activation vectors before the FSM re-enters S_STREAM, or the FSM will stall in S_STREAM until act_valid is asserted.

Example: tile_count=1, act_tile_count=4, vector_count=8. The array will compute 4 batches of 8 activation vectors against the same weight tile, producing 32 output vectors total before returning to S_IDLE.

## 5.3 Multi-Tile Operation

For tile_count > 1: After each act_tile_count completion, tile_cnt is incremented. If tile_cnt < tile_count, the FSM re-enters S_LOAD_WEIGHTS (if the next tile is buffered) or S_STREAM (if the weight tile is not yet buffered but streaming should begin). The weight prefetch mechanism (wgt_prefetch_start, V18.3) overlaps the next tile's streaming into the weight buffer with the current tile's computation, hiding the weight load latency.

# 6. Verification Environment Overview

## 6.1 UVM Testbench Architecture

The verification environment follows a standard UVM layered architecture targeting accel_top_v18 as the DUT. The testbench is organised into the following components:

### 6.1.1 Agents

- AXI4-Lite Master Agent: Drives the AXI4-Lite CSR interface. Includes a sequencer, driver, and monitor. The driver implements the AW+W simultaneous vs. separate handshake paths. The monitor captures all CSR transactions for scoreboard comparison.

- AXI4-Stream Weight Agent (Master): Drives s_axis_weight_*. Generates weight_tuser_t metadata including correct sparsity_mode and sparse_mask encoding. Supports programmable beat packing and tlast insertion.

- AXI4-Stream Activation Agent (Master): Drives s_axis_act_*. Supports vector-granularity tlast insertion and configurable inter-vector gap injection to test backpressure.

- AXI4-Stream Result Agent (Slave): Monitors m_axis_result_*. Collects result beats, reassembles into 32-bit result vectors, and forwards to the scoreboard. Applies random tready de-assertion for backpressure testing.

- IRQ Monitor: Tracks irq_out transitions and maps to CSR_IRQ_STATUS. Verifies that IRQ sources fire at the expected computation events.

### 6.1.2 Scoreboard and Reference Model

The scoreboard hosts a software reference model that mirrors the DUT computation:

- Weight and activation inputs captured by agent monitors are forwarded to the reference model.

- The model performs IEEE-compliant INT8 × INT8 multiply-accumulate (two per PE per phase) and applies the same sparsity index selection logic as the RTL.

- Post-processing (bias, scale, shift, saturation) is applied using the programmed CSR values.

- Expected output vectors are compared cycle-accurately against the DUT's result stream. Mismatches are flagged as UVM_ERROR.

- [TODO] Reference model for dense mode two-phase accumulation and PIPELINE_LATENCY offset calibration.

## 6.2 SystemVerilog Assertions (SVA)

### 6.2.1 Protocol Checkers

- s_axis_weight_tvalid |-> ##[1:$] s_axis_weight_tready: TREADY must eventually be asserted (liveness)

- !s_axil_bvalid || $past(s_axil_awvalid && s_axil_awready): Write response only after write address

- s_axil_rvalid |-> s_axil_rdata !== 'x: No X-propagation on read data

- s_axis_weight_tuser[7:0] !== 8'hxx: TUSER must be defined on valid beats

### 6.2.2 FSM Checkers

- state == S_LOAD_WEIGHTS |-> load_en: load_en must be asserted during weight loading

- state == S_IDLE |-> !compute_en && !drain_en: No compute activity in idle

- state == S_STREAM |-> compute_en: compute_en asserted throughout streaming (modulo backpressure)

- $rose(done) |=> state == S_IDLE: Done implies FSM returns to IDLE

- abort |=> state == S_ERROR: Abort leads to error state

### 6.2.3 Data Integrity Checkers

- result_valid |-> !$isunknown(psum_out): No X in outputs when valid is asserted

- load_en && load_row_sel < ROWS_P: Row selection always in-range

- col_valid == '1 |-> ##[0:2] ofifo_valid_out: Valid array output must reach the FIFO within pipeline budget

## 6.3 Functional Coverage

### 6.3.1 Covergroups

- cg_sparsity_modes: Crosses all four sparsity_mode_e values with enable to ensure each mode is exercised.

- cg_vector_counts: Bins for vector_count ∈ {1, 2, 4, 8, ROWS_P, ROWS_P+1, max} to test boundary conditions.

- cg_tile_counts: Bins for tile_count ∈ {1, 2, max} and act_tile_count ∈ {1, 2, 4}.

- cg_postproc_ops: Crosses all postproc_op_e values with round_en and sat_en flags.

- cg_backpressure: Captures scenarios where m_axis_result_tready is de-asserted for 0, 1–5, >5 consecutive cycles.

- cg_irq_sources: Ensures each IRQ source fires at least once with its enable mask set.

- cg_perf_counters: Verifies non-zero values in all five counters across at least one test.

### 6.3.2 Cover Properties

- cover property (@clk disable iff (!rst_n) (state == S_STREAM) ##[1:$] (state == S_DRAIN)): Streaming → draining transition

- cover property (@clk disable iff (!rst_n) (state == S_DONE && act_tile_cnt < act_tile_count) ##1 (state == S_STREAM)): Weight reuse activation

- cover property (@clk disable iff (!rst_n) (wgt_prefetch_start) ##[1:$] (state == S_LOAD_WEIGHTS)): Prefetch followed by load

- cover property (@clk disable iff (!rst_n) (parity_error)): Weight parity error injection and detection

- cover property (@clk disable iff (!rst_n) (zero_weight_detected) throughout (compute_en [*8])): Sustained zero-weight detection across 8 cycles